Window Control STM32 WiFi Technical Documentation

# Document purpose

This document describes the STM32 WiFi implementation of the Window Control project. It is written in third person for technical handover, internal documentation, and future maintenance. Diagram sections are intentionally left as headings so the final captures or drawings can be inserted later.

# Table of Contents

1. Hardware Documentation

1.1 Hardware Overview

1.2 Bill of Materials

1.3 Structural System Diagram (Components)

1.4 Connection Diagrams

1.5 GPIO and Signal Allocation

1.6 Power and Signal Domains

2. Software Documentation

2.1 File Structure

2.2 Class and Function Structure

2.3 State Machine and Thread Functionality

2.4 Manual Movement Functionality

2.5 Automatic Solar Tracking Functionality

2.6 GPS Functionality

2.7 WiFi GUI Functionality

2.8 Power-Consumption Characterization

2.9 Low-Power Functionality

2.10 XZ Displacement Validation

2.10.1 XZ Displacement Order

2.10.2 Phi/Theta Input Angle Order

2.11 Automatic Solar Tracking Validation

2.12 MODBUS RTU Functionality (TFG A. Carrasco)

2.13 HMI Display Functionality (TFG N. Aguero)

3. Development and Maintenance Procedures

3.1 Adding Project Files and Fixing Include/Linker Errors

3.1.1 Example: hello Shared Variable Declaration

3.2 Regenerating html_pages.h

3.3 Creating and Invoking a FreeRTOS Task

3.4 Adding a New Status Field to the Web Interface

# 1. Hardware Documentation

## 1.1 Hardware Overview

The hardware platform is based on an STM32U575 microcontroller board, an ST67W6X WiFi coprocessor expansion, external stepper-driver electronics, limit switches, and a GPS receiver. The STM32U575 is the main controller and executes all application logic. The ST67W6X module provides WiFi connectivity and the HTTP interface. The motor stage is controlled through STEP, DIR, and ENABLE signals. Limit switches provide the homing and motion-protection feedback.

| Element | Role |
| --- | --- |
| NUCLEO-U575ZI-Q / STM32U575 | Main application controller. It runs FreeRTOS, the state machine, movement logic, RTC, storage, GPS handling, and GPIO control. |
| ST67W6X / ST67W611M WiFi expansion | WiFi coprocessor used by the HTTP server. It communicates through SPI and is controlled through CHIP_EN and SPI_RDY. |
| CNC Shield V3 / stepper-driver carrier | Motor-driver carrier used to distribute STEP, DIR, and ENABLE signals. |
| Stepper motors | Six motors are represented in firmware: four vertical motors and two horizontal carriage motors. |
| Limit switches | End-stop sensors used by homing and movement protection. They are monitored through EXTI and cached in software. |
| NEO-6M GPS receiver | UART GPS receiver used to obtain NMEA date/time data for RTC synchronization. |
| RTC/LSE clock source | Low-speed clock source used to keep local time once the RTC has been configured. |

## 1.2 Bill of Materials

| Item | Recommended part / description | Notes |
| --- | --- | --- |
| Main controller board | ST NUCLEO-U575ZI-Q or compatible STM32U575 board | The firmware is generated for STM32U575ZITxQ and STM32CubeIDE. |
| WiFi expansion | ST67W6X / ST67W611M compatible X-NUCLEO expansion | Required for the current HTTP server application. |
| Motor shield | CNC Shield V3 or equivalent STEP/DIR/ENABLE carrier | Used to distribute stepper-driver control signals. |
| Stepper drivers | A4988/DRV8825-compatible modules or selected driver modules | Driver current must be configured externally according to motor requirements. |
| Stepper motors | Six stepper motors for vertical and horizontal mechanisms | Four vertical motors and two horizontal motors are represented in firmware. |
| Limit switches | Six end-stop switches | The current implementation assumes EXTI rising/falling edge detection with software state caching. |
| GPS receiver | NEO-6M GPS module or equivalent NMEA UART receiver | Used for UTC date/time extraction and RTC synchronization. |
| DC-DC converter / logic supply | Stable 5 V logic rail for external driver logic if required | Ground must be common between controller and external logic. |
| Motor power supply | External DC supply sized for all active motors | Motor power must not be sourced from the STM32 board. |
| Power switches | Logic-level MOSFETs or dedicated load switches | Recommended for future low-power control of GPS, motor logic, or motor power. |
| Connectors and wiring | Morpho/Arduino headers, jumper wires, screw terminals | All wiring should be checked for shorts before power-up. |

## 1.3 Structural System Diagram (Components)

[Diagram placeholder. The system component diagram should be inserted here.]

## 1.4 Connection Diagrams

[Diagram placeholder. The final wiring and pin-to-pin connection diagrams should be inserted here.]

## 1.5 GPIO and Signal Allocation

| Function group | Signal | STM32 pin / port |
| --- | --- | --- |
| Motor enable | Horizontal_ENABLE | PA3 |
| Motor enable | Vertical_ENABLE | PF11 |
| ST67W6X SPI | SPI_CLK / SPI_MISO / SPI_MOSI / SPI_CS | PA5 / PA6 / PA7 / PD14 |
| ST67W6X control | SPI_RDY / CHIP_EN | PE13 / PE11 |
| Debug/log UART | USART1_TX / USART1_RX | PA9 / PA10 |
| Vertical end stops | YLY, YLE, YRE, YRI | PB1, PB2, PB4, PB5 |
| Horizontal end stops | ZL, ZR | PB14, PB15 |
| Vertical motor outputs | MXLI_X_STEP/DIR | PE0 / PG14 |
| Vertical motor outputs | MXLE_Y_STEP/DIR | PG8 / PD10 |
| Vertical motor outputs | MXRI_Z_STEP/DIR | PG5 / PG7 |
| Vertical motor outputs | MXRE_A_STEP/DIR | PG6 / PG4 |
| Horizontal motor outputs | ZL_A_STEP/DIR | PD11 / PE10 |
| Horizontal motor outputs | ZR_Z_STEP/DIR | PD13 / PD12 |
| User LEDs | LED_RED / LED_GREEN / LED_BLUE | PG2 / PC7 / PB7 |
| User button | USER_BUTTON | PF12 |

## 1.6 Power and Signal Domains

- The STM32 logic domain operates at 3.3 V, and GPIO-connected devices must be compatible with this level.

- The CNC shield and stepper-driver logic require a stable logic supply. The motor supply must be external and sized for the mechanical load.

- The GPS module should be powered through a controlled switch in the final low-power design.

- The WiFi coprocessor is controlled through CHIP_EN, although final current reduction must be validated electrically.

- Continuity between 3V3, 5V, VIN, and GND should be checked before applying power after wiring changes.

# 2. Software Documentation

## 2.1 File Structure

| Path | Purpose |
| --- | --- |
| Core/Src and Core/Inc | Application logic, STM32 HAL user code, movement, GPS, state machine, automatic mode, storage, and generated peripheral setup. |
| Appli/App | ST67W6X HTTP server layer, web request handling, application startup hook, and generated HTML page data. |
| Html | Source HTML used to regenerate html_pages.h. |
| ST67W6X | ST WiFi coprocessor application and target support files. |
| Middlewares | FreeRTOS, ST67W6X network driver, LwIP/mbedTLS components, and related third-party middleware. |
| Drivers | STM32 HAL and CMSIS support files. |
| STM32CubeIDE | Build metadata, linker script, and project linked-resource definitions. |

## 2.2 Class and Function Structure

The firmware is organized as C/C++ modules rather than object-oriented classes. Each module exposes a small set of functions and shares runtime data through global_structs when necessary.

| Module | Main entry points | Responsibility |
| --- | --- | --- |
| solar_app | SolarApp_Start(), SolarApp_Process() | Starts application-level services, initializes movement resources, requests homing, initializes the FSM, and requests GPS time synchronization. |
| state_machine | initFSM(), fsmPostEvent(), fsmProcess(), fsmGetState() | Owns states, events, transitions, periodic automatic mode, and application event queue. |
| httpserver | http_process_response(), http_apply_*() | Maps browser requests to configuration changes, FSM events, JSON status, and error pages. |
| movement_task | initMovementTask(), requestMove(), requestHome() | Serializes blocking movement and homing commands through a FreeRTOS queue. |
| movement | move(), GoHomePair(), movementLimitSwitchUpdateFromExti() | Generates STEP/DIR/ENABLE signals and handles end-stop-aware motion. |
| gps | GPS_Task(), GPS_Task_RequestTimeSync(), GPS_Task_SyncTimeNow() | Parses NMEA data and synchronizes the RTC. |
| storage | loadData(), saveData(), saveRtcTime(), saveManualTime() | Stores and restores configuration, state, position, and time. |
| autoMode | autoMode() | Runs one automatic solar-tracking iteration. |
| manual_mode | manualModeGoto() | Loads manual X/Z target values. |
| eph_input_mode | ephInputMode() | Calculates movement targets from submitted azimuth/elevation values. |

## 2.3 State Machine and Thread Functionality

| State | Meaning |
| --- | --- |
| STDBY | Idle state. The system accepts configuration, homing, manual mode, ephemeris mode, GPS sync, and automatic-mode toggling. |
| AUTO_MODE | Automatic tracking state. The FSM executes autoMode() periodically every five seconds. |
| MANUAL | Manual positioning state. The web interface can submit X/Z targets. |
| EPH_INPUT | Ephemeris input state. The web interface can submit azimuth/elevation values. |
| Thread/task | Responsibility |
| defaultTask | CubeMX-generated default FreeRTOS task. It remains minimal. |
| gpsTask | Runs GPS_Task(), parses NMEA frames, and services pending RTC synchronization requests. |
| FSM_Task | Consumes FSM events, changes states, saves time periodically, and runs autoMode() in AUTO_MODE. |
| Movement_Task | Executes move and homing commands without blocking the HTTP or FSM task. |
| HTTP client tasks | Handle individual HTTP client requests from the ST HTTP server. |
| WiFiOffTest | Experimental task used to test ST67W6X shutdown behavior while keeping the STM32 alive. |

## 2.4 Manual Movement Functionality

Manual movement receives X/Z target values from the web form. The HTTP layer parses the values and stores them in g_x_target and g_z_target. The FSM receives submit_manual_goto, calls manualModeGoto(), and posts requestMove() to the movement task. The movement task copies the target into a queue message so later web changes cannot alter a command already accepted.

## 2.5 Automatic Solar Tracking Functionality

Automatic mode runs periodically while the FSM is in AUTO_MODE. It reads configuration and local time, calculates solar position, computes the angle of incidence, interpolates the target position, and requests movement only when the displacement is large enough to avoid accumulated small-step error.

## 2.6 GPS Functionality

The GPS task continuously parses NMEA data. When a valid time/date sentence is available and synchronization is requested, the firmware converts UTC to local time according to the configured offset logic and loads the STM32 RTC. The web interface can request synchronization through the SYNC TIME control.

## 2.7 WiFi GUI Functionality

The WiFi GUI is served by the ST67W6X HTTP server application. HTML source is maintained in the Html folder and must be regenerated into html_pages.h. The browser submits GET requests for configuration, manual mode, ephemeris mode, automatic-mode toggle, homing, GPS synchronization, and status polling.

## 2.8 Power-Consumption Characterization

Power characterization should be documented by measuring the controller alone, the controller with WiFi, the GPS receiver, the motor-driver logic, and active motor movement separately. Each measurement should record supply voltage, average current, operating mode, firmware version, and connected peripherals.

## 2.9 Low-Power Functionality

Low-power operation is handled through FreeRTOS tickless idle and ST low-power configuration. Movement and automatic-mode calculations temporarily block tickless entry while deterministic timing is required. Further reductions require controlled power switching for external loads and a validated WiFi coprocessor shutdown path.

## 2.10 XZ Displacement Validation

XZ displacement validation confirms that submitted or calculated target positions produce the expected mechanical motion. The validation must compare g_x_target/g_z_target, completed g_x_val/g_z_val, motor direction, and measured displacement.

### 2.10.1 XZ Displacement Order

This subsection should document the order in which X and Z movement commands are generated, executed, and validated. It should also record whether axes move independently or sequentially during each test case.

### 2.10.2 Phi/Theta Input Angle Order

This subsection should document the expected order and sign convention of phi/theta input angles before they are converted into AOI and interpolation targets.

## 2.11 Automatic Solar Tracking Validation

Automatic solar tracking validation should compare local RTC time, GPS time, configured location, calculated azimuth/elevation, AOI output, interpolation output, and final X/Z target. The result should be checked against expected panel movement.

## 2.12 MODBUS RTU Functionality (TFG A. Carrasco)

[Reserved section. This section should document the future MODBUS RTU integration delivered by the referenced work.]

## 2.13 HMI Display Functionality (TFG N. Aguero)

[Reserved section. This section should document the future HMI display integration delivered by the referenced work.]

# 3. Development and Maintenance Procedures

This chapter groups practical procedures that are required during firmware maintenance. It is intended to reduce repeated setup errors when files, HTML pages, tasks, or status fields are added.

## 3.1 Adding Project Files and Fixing Include/Linker Errors

- Headers should be placed in Core/Inc and source files should be placed in Core/Src unless the module belongs to the ST application layer.

- A missing header error normally means the include path is wrong or the header is not located under an included folder such as ../../Core/Inc.

- An undefined reference error normally means the source file is not being compiled or a C/C++ name-mangling boundary is missing.

- When C code calls a C++ implementation, the public header should wrap declarations with extern "C" guards.

#ifdef __cplusplus

extern "C" {

#endif

void exampleFunction(void);

#ifdef __cplusplus

}

#endif

- If STM32CubeIDE does not show or compile a file, the project linked resources should be checked in STM32CubeIDE/.project. Existing project entries can be used as templates for Application/User/Core links.

- After adding files manually, the project should be refreshed in STM32CubeIDE and rebuilt from a clean state if the incremental build still uses stale metadata.

## 3.1.1 Example: hello Shared Variable Declaration

This example documents the common build failure where Core/Src/state_machine.c uses hello but the compiler reports error: 'hello' undeclared. The source file may exist, but the compilation unit still needs a visible declaration before the variable can be used.

For a shared global variable, the header must declare the symbol with extern and exactly one source file must define storage for it. In this example, Core/Inc/hello.h declares extern int hello; and Core/Src/hello.c defines int hello = 0; . The file that uses the variable includes hello.h.

/* Core/Inc/hello.h */

#ifndef HELLO_H

#define HELLO_H

extern int hello;

#endif /* HELLO_H */

/* Core/Src/hello.c */

int hello = 0;

If the compile error disappears but the linker later reports undefined reference to hello, the declaration is visible but the source file that defines the variable is not being compiled into the project. In CubeIDE, the file must be visible as a linked resource in STM32CubeIDE/.project.

<link>

<name>Application/User/Core/hello.c</name>

<type>1</type>

<locationURI>PARENT-1-PROJECT_LOC/Core/Src/hello.c</locationURI>

</link>

The .cproject file normally does not list each application source file one by one in this project. It stores compiler options, include paths, symbols, linker settings, and build configuration. It should only be changed for this type of issue when a new include directory is required, for example if headers are moved outside ../../Core/Inc.

After adding files to .project, CubeIDE should refresh the project and perform a clean build so the generated Debug/Application/User/Core/subdir.mk and objects.list are rebuilt with the new source file.

## 3.2 Regenerating html_pages.h

The web interface source is edited in Html/index_httpserver.html. The generated C header used by the firmware is Appli/App/html_pages.h. After each HTML change, the conversion script must be executed from the ST67W6X_HTTP_Server folder.

cd C:\Users\ISI\Documents\Alvaro\Window_Control\Window_Control\STM32_wifi\STM32Workspace\ST67W6X_HTTP_Server

py -3 Html\html_to_h.py --config Html\conf.txt --output Appli\App

- The command should regenerate html_pages.h in Appli/App.

- If the web page does not change after flashing, the generated header should be checked before debugging HTTP logic.

## 3.3 Creating and Invoking a FreeRTOS Task

Long-running or blocking code should run in its own task. HTTP handlers and FSM transitions should post events or queue commands instead of generating long delays or motor pulses directly.

static TaskHandle_t exampleTaskHandle = NULL;

static void exampleTask(void *argument)

{

(void)argument;

for (;;)

{

/* Task work goes here. */

vTaskDelay(pdMS_TO_TICKS(1000));

}

}

void exampleTaskStart(void)

{

if (exampleTaskHandle == NULL)

{

(void)xTaskCreate(exampleTask,

"Example_Task",

1024U,

NULL,

tskIDLE_PRIORITY + 1U,

&exampleTaskHandle);

}

}

- The (void)argument line intentionally marks an unused task argument as handled. This avoids compiler warnings when the task receives no parameter.

- If a task should execute once, it should call vTaskDelete(NULL) before returning.

- If a task receives commands, a FreeRTOS queue should be preferred over shared flags when command order matters.

## 3.4 Adding a New Status Field to the Web Interface

- The variable should first be declared in the correct module. Shared runtime variables are usually declared in global_structs.h and defined in global_structs.c.

- The /status JSON response is built in httpserver.c. A new JSON field must be added to the snprintf format string and to the argument list.

- If the status JSON grows, the allocated response buffer size must be reviewed.

- The HTML source in Html/index_httpserver.html must add a visible element with an id for the new field.

- The JavaScript status polling code must read the JSON field and update that element.

- After editing HTML, html_pages.h must be regenerated with the command documented in section 3.2.

Example JSON field: "new_value":%d

Example HTML field: <span id="new_value">---</span>

Example JavaScript update: setText("new_value", d.new_value);