Window Control STM32 WiFi Technical Documentation

# Document purpose

This document describes the hardware and software structure of the STM32 WiFi implementation of the Window Control project. It is written for maintenance and future development. The document intentionally does not include structural or wiring diagrams, because those diagrams are maintained separately.

# Project scope

- The target firmware is located in STM32_wifi/STM32Workspace/ST67W6X_HTTP_Server.

- The project migrates the ESP32 window-control application to an STM32U575 platform with an ST67W6X WiFi coprocessor.

- The firmware exposes a WiFi HTTP interface for configuration, manual positioning, ephemeris input, automatic mode, homing, GPS time synchronization, and status monitoring.

- The implementation uses FreeRTOS tasks and queues to keep HTTP handling, state transitions, GPS parsing, and motor movement separated.

# Hardware Documentation

## Main hardware elements

| Element | Role |
| --- | --- |
| NUCLEO-U575ZI-Q / STM32U575 | Main application controller. It runs the FreeRTOS application, HTTP server integration, state machine, movement logic, RTC, storage logic, and GPIO control. |
| X-NUCLEO / ST67W6X WiFi coprocessor | WiFi coprocessor used by the ST HTTP server application. It communicates with the STM32 through SPI and is controlled through CHIP_EN and SPI_RDY lines. |
| CNC Shield V3 / motor driver carrier | Motor-driver carrier used to generate STEP, DIR, and ENABLE signals for the mechanical axes. |
| Stepper motors | Actuators for horizontal and vertical window/panel positioning. Four vertical motors and two horizontal carriage motors are handled by the firmware. |
| Limit switches | End-stop sensors used by homing and safety logic. The firmware keeps an interrupt-updated cache of the active switches. |
| NEO-6M GPS receiver | UART GPS receiver used to obtain valid NMEA date/time and synchronize the RTC. |
| External motor supply | 12 V power source for the motor drivers and mechanical actuation stage. Logic and motor power should be treated as separate domains. |
| RTC/LSE clock source | Low-speed timing source used by the STM32 RTC for local date/time persistence between updates. |

## Power and signal domains

- The STM32 logic domain operates at 3.3 V. GPIO signals connected to the STM32 must be compatible with this voltage level.

- The CNC shield and stepper require 5 V provided by the DC-DC converter.

- The GPS module must be connected through a power switch to lower consumption.

- The ST67W6X module is controlled by CHIP_EN. Software tests attempted to reduce consumption by stopping the SoftAP and deinitializing the WiFi coprocessor.

## Current GPIO allocation

| Function group | Signal | STM32 pin / port |
| --- | --- | --- |
| Motor enable | Horizontal_ENABLE | PA3 |
| Motor enable | Vertical_ENABLE | PF11 |
| ST67W6X SPI | SPI_CLK | PA5 |
| ST67W6X SPI | SPI_MISO | PA6 |
| ST67W6X SPI | SPI_MOSI | PA7 |
| ST67W6X SPI | SPI_CS | PD14 |
| ST67W6X control | SPI_RDY | PE13 |
| ST67W6X control | CHIP_EN | PE11 |
| Debug/log UART | USART1_TX | PA9 |
| Debug/log UART | USART1_RX | PA10 |
| Vertical end stops | YLY, YLE, YRE, YRI | PB1, PB2, PB4, PB5 |
| Horizontal end stops | ZL, ZR | PB14, PB15 |
| Vertical motor outputs | MXLI_X_STEP/DIR | PE0 / PG14 |
| Vertical motor outputs | MXLE_Y_STEP/DIR | PG8 / PD10 |
| Vertical motor outputs | MXRI_Z_STEP/DIR | PG5 / PG7 |
| Vertical motor outputs | MXRE_A_STEP/DIR | PG6 / PG4 |
| Horizontal motor outputs | ZL_A_STEP/DIR | PD11 / PE10 |
| Horizontal motor outputs | ZR_Z_STEP/DIR | PD13 / PD12 |
| User interface LEDs | LED_RED, LED_GREEN, LED_BLUE | PG2, PC7, PB7 |
| User input | USER_BUTTON | PF12 |

# Bill of Materials

| Item | Recommended part / description | Notes |
| --- | --- | --- |
| Main controller board | ST NUCLEO-U575ZI-Q or compatible STM32U575 board | The firmware is generated for STM32U575ZITxQ and STM32CubeIDE. |
| WiFi expansion | ST67W6X / ST67W611M compatible X-NUCLEO expansion | Required for the current HTTP server application. |
| Motor shield | CNC Shield V3 or equivalent STEP/DIR/ENABLE carrier | Used to distribute stepper driver signals. |
| Stepper drivers | A4988/DRV8825-compatible modules or selected driver modules | Driver current must be configured externally according to the motors. |
| Stepper motors | Six stepper motors for vertical and horizontal mechanisms | Four vertical motors and two horizontal motors are represented in firmware. |
| Limit switches | Six end-stop switches | EXTI rising/falling edge trigger detection and pull down |
| GPS receiver | NEO-6M GPS module or equivalent NMEA UART receiver | Used for UTC date/time extraction and RTC synchronization. |
| Motor power supply | External DC supply sized for all active motors | Must supply motor drivers directly, not through the STM32 board. |
| Logic power supply | Stable 5 V rails as required by selected boards | Ground must be common between controller and motor logic. |
| Connectors and wiring | Morpho/Arduino headers, jumper wires, screw terminals | Final wiring should be strain-relieved and checked for shorts before power-up. |
| Power switches | Logic-level MOSFET | Low-power control of GPS, driver logic, or motor power. |

# Software Documentation

## Firmware structure

| Path | Purpose |
| --- | --- |
| Core/Src and Core/Inc | Application logic, STM32 HAL user code, movement, GPS, state machine, automatic mode, storage, and generated peripheral setup. |
| Appli/App | ST67W6X HTTP server application layer, web request handling, application startup hook, and generated HTML page data. |
| Html | Source HTML used to regenerate html_pages.h through the ST html_to_h.py flow. |
| ST67W6X | ST WiFi coprocessor application and target support files. |
| Middlewares | FreeRTOS, ST67W6X network driver, LwIP/mbedTLS components, and related third-party middleware. |
| Drivers | STM32 HAL and CMSIS device support. |
| STM32CubeIDE | Build metadata, linker script, and project file links used by STM32CubeIDE. |

## Important application files

| File | Responsibility |
| --- | --- |
| solar_app.c / solar_app.h | Starts the application layer by initializing movement resources, requesting homing, initializing the FSM, and requesting GPS time synchronization. |
| state_machine.c / state_machine.h | Owns the application state machine, event queue, periodic automatic mode execution, and state persistence. |
| httpserver.c / httpserver.h | Handles HTTP requests from the web interface, parses form data, updates global configuration, posts FSM events, and serves status JSON. |
| movement.cpp / movement.h | Generates motor GPIO pulses, direction signals, homing phases, end-stop checks, and low-level movement behavior. |
| movement_task.cpp / movement_task.h | Runs blocking movement and homing commands in a dedicated FreeRTOS task using a command queue. |
| gps.cpp / gps.h | Reads NMEA data from UART, parses valid date/time, computes local time, and synchronizes the RTC on request. |
| storage.c / storage.h | Persists configuration, state, position, and time information in non-volatile memory. |
| autoMode.cpp / autoMode.h | Runs one automatic tracking iteration using configuration, local time, solar-position calculations, AOI calculation, interpolation, and movement target generation. |
| manual_mode.cpp / manual_mode.h | Converts web manual X/Z target requests into global movement targets. |
| eph_input_mode.cpp / eph_input_mode.h | Processes manually provided azimuth/elevation values and computes movement targets through AOI/interpolation logic. |
| spa.c, spa_func.cpp / spa_func.h | Provide solar position algorithm support for automatic tracking. |
| aoicalc.c, aoicalc_func.cpp / aoicalc_func.h | Calculate angle-of-incidence values used by target interpolation. |
| interpolation.c, interpolation_func.cpp / interpolation_func.h | Convert calculated angular values into target positions. |
| global_structs.c / global_structs.h | Declare shared runtime variables such as configuration, positions, targets, state flags, and status values. |

## Class and function structure

The project is written mostly in C and C++ modules rather than object-oriented classes. The main architectural units are therefore modules, public functions, and shared data structures.

| Module | Public functions / entry points | Description |
| --- | --- | --- |
| Application startup | SolarApp_Start(), SolarApp_Process() | Initializes movement, homing, FSM, and GPS sync. SolarApp_Process remains as a compatibility polling hook. |
| FSM | initFSM(), fsmPostEvent(), fsmProcess(), changeState(), fsmGetState(), stateToText() | Creates the FSM queue/task, translates events into states, launches mode routines, and exposes current state to HTTP status. |
| Movement task | initMovementTask(), requestMove(), requestHome(), movementIsBusy() | Serializes homing and movement commands so long pulse generation does not block HTTP or FSM execution. |
| Movement low level | initMotors(), move(), GoHomePair(), movementLimitSwitchUpdateFromExti(), movementAnyLimitSwitchActive() | Controls enable, step, direction, homing, end-stop cache updates, and safety checks. |
| GPS | GPS_Task(), GPS_Task_RequestTimeSync(), GPS_Task_SyncTimeNow() | Continuously parses NMEA data and services pending time synchronization requests. |
| Storage | loadData(), saveData(), saveRtcTime(), saveManualTime() | Stores and restores persistent configuration, position, state, and time values. |
| Automatic mode | autoMode() | Executes one automatic tracking iteration and updates movement targets when target displacement is significant. |
| Manual mode | manualModeGoto() | Loads manual X/Z targets from the web request into the target variables. |
| Ephemeris input mode | ephInputMode() | Uses submitted azimuth/elevation inputs to calculate target X/Z positions. |
| HTTP interface | http_process_response(), http_apply_config_submit(), http_apply_manual_datetime(), http_apply_eph_submit(), http_apply_manual_goto() | Maps browser requests to application data, FSM events, status JSON, and error pages. |

# State machine functionality

## States

| State | Meaning |
| --- | --- |
| STDBY | Default idle state. The system accepts configuration, homing, manual entry, ephemeris entry, GPS sync, and automatic-mode toggling. |
| AUTO_MODE | Automatic tracking state. The FSM periodically executes autoMode() every five seconds and posts movement requests when necessary. |
| MANUAL | Manual positioning state. The web interface can submit explicit X/Z targets and the FSM forwards them to movement_task. |
| EPH_INPUT | Ephemeris input state. The web interface can submit azimuth/elevation values and the FSM calculates target movement from them. |

## Events

| Event | Source / effect |
| --- | --- |
| begin_manual | Web request selects manual mode. |
| submit_manual_goto | Web request submits X/Z targets; the FSM calls manualModeGoto() and requestMove(). |
| end_manual | Web request leaves manual mode and returns to STDBY. |
| begin_eph_input | Web request selects ephemeris input mode. |
| submit_eph_input | Web request submits azimuth/elevation; the FSM runs ephInputMode() and may call requestMove(). |
| end_eph_input | Web request leaves ephemeris input mode and returns to STDBY. |
| submit_home | Web request asks for homing; movement_task receives CMD_HOME. |
| toggle_auto_mode | Web request toggles STDBY and AUTO_MODE. |

## FSM behavior

1.	The FSM task owns the application state and receives events through a FreeRTOS queue.

2.	HTTP handlers do not execute long operations directly. They parse input, update variables, and post events.

3.	Automatic mode is periodic. While the state is AUTO_MODE, autoMode() is executed every 5000 ms.

4.	Movement and homing are not executed inside the FSM task. The FSM posts commands to movement_task.

- State changes are mirrored into legacy flags such as auto_on so older modules can keep reading the expected variables.

- The FSM periodically saves RTC/time-related data so the system can recover meaningful state after reset or power loss.

# Thread and task functionality

| Task/thread | Created by | Main responsibility |
| --- | --- | --- |
| defaultTask | CubeMX / app_freertos.c | Default FreeRTOS task generated by CubeMX. It remains minimal in this application. |
| gpsTask | app_freertos.c | Runs GPS_Task(), parses NMEA frames, and synchronizes RTC when requested. |
| FSM_Task | state_machine.c | Processes application events, state transitions, storage timing, and automatic-mode periodic execution. |
| Movement_Task | movement_task.cpp | Executes blocking movement and homing commands from a queue, updates final position, and protects pulse timing from low-power entry. |
| HTTP client tasks | httpserver.c | Created by the ST HTTP server socket loop to process client requests. |
| WiFiOffTest | controled_shutdown.c | Experimental one-shot task used to test controlled WiFi module shutdown without stopping the STM32U575. |

## Data flow summary

- The browser sends HTTP GET form requests to httpserver.c.

- httpserver.c validates the request, updates global configuration or target variables, and posts an FSM event.

- state_machine.c consumes the event and decides whether to change state or launch a mode routine.

- manual_mode.cpp, eph_input_mode.cpp, and autoMode.cpp update g_x_target and g_z_target.

- movement_task.cpp captures target values into queue messages, executes movement, then updates g_x_val and g_z_val after motion completes.

- gps.cpp continuously updates time data from NMEA and loads the RTC when synchronization is requested.

- storage.c persists configuration, state, position, and time data for startup recovery.

# Operational notes

- The HTML source must be regenerated into html_pages.h through the ST html_to_h.py flow before the web interface changes are available in firmware.

- Blocking motor pulse generation must remain outside HTTP callbacks and outside the FSM transition code.

- Low-power experiments must be isolated from movement and WiFi tests. Movement commands disable tickless sleep while pulses are generated.

- The final hardware should include a controlled power strategy for GPS, motor logic, motor power, and the WiFi coprocessor if low average consumption is required.

- All wiring changes should be validated with continuity checks between 3V3, 5V, VIN, and GND before applying power.